| R | DATE | DESCRIPTION |
| --- | --- | --- |
| 0 | 30/04/2018 | First issue |

| CHECK | APPROVAL |
| --- | --- |
| RSGQ | DG |

# Technical information of the partly completed machine

| Name: Product: Model: Serial number: | Punto_TOX Q.M. - Quasi-machine Punto_TOX 20251177 |
| --- | --- |
| Review: Year of manufacture: Brand: Intended use: Description: | 00 - 15/03/2017 12.17.02 2025 Dietronic s.r.l. Partly completed machine for point lubrication ThePunto_TOXseries is a partly completedmachine |

| Created by: | Inspected by: | Validated by: | Date: | Observations: |
| --- | --- | --- | --- | --- |
| Boera Stefano | Cosentino Dario | Tavazzi Davide | 06/8/2025 |  |

Risk Assessment overall status

Directive 2006/42/EC of European Parliament and the Council of May 17t2006 on machinery and amendments of Directive 95/16/EC

| Manufacturer: Dietronic s.r.l. |  |  |  |
| --- | --- | --- | --- |
| Via Madre Teresa di Calcutta 13 |  | 26866 | Lodi |
| Address |  | postal code | Province |
| Sant’Angelo Lodigiano |  | Italia |  |
| City |  | Country |  |
| Machine: |  |  |  |
| Partly completed - machine for point lubrication |  |  |  |
| Intended use |  |  |  |
| Punto_TOX | 20251177 | 2025 | 00 - 15/03/2017 |
| Model | Serial number | Year of manufacture | Rewiew |

INDEX

- MATERIALSANDPRODUCTS	5

- MACHINE DESIGN FOR THE PURPOSEOFHANDLING	5

- SAFETY AND RELIABILITY OF THESTOPSYSTEMS	6

- SELECTION OF THE CONTROL OROPERATINGMODE	6

- ELECTRICALSPECIFICATIONS	7

- NOISE	7

- INFORMATION ANDINFORMATIONDEVICES	8

- MACHINELABELING	8

- SIGNS	9

- ELECTRICALCOMPONENTSINSPECTION	10

- PNEUMATIC AND OIL-HYDRAULICCOMPONENTSINSPECTION	11

- MECHANICALCOMPONENTSINSPECTION	12

| 1. Materials and products The materials or products used for the construction of the machine or originated during its use must not represent a risk for people’s safety and health. In particular, in case of fluids usage, the machine must be designed and built to prevent risks due to filling, use, recovery and evacuation. Standards: ISO/TR 14121-2 and EN ISO 12100 Scheduled action: Visual inspection of the correct construction of the machine frame and the cutting-edge assembly of all parts including the mechanical and pneumatic components. Outcome: OK Date: 08/8/2025 Notes: Possible intervention method: Closing date: ……………………………………………………… | 2. Machine design for the purpose of handling The machine, or each of its different components, must be handled and transported safely. The machine is equipped with accessories that facilitate a safe handling with a lifting device or has a shape that allows the lifting means to adapt easily. Standards: ISO/TR 14121-2, EN ISO 12100, EN 1005-2 and EN 1005-4 Scheduled action: Visual inspection of the correct construction of the machine frame. Lifting and handling tests of the machine and all its components. Outcome: OK Date: 08/8/2025 Notes: Possible intervention method: Closing date: ……………………………………………………… |
| --- | --- |

| 3. Safety and reliability of the stop systems The control systems must be designed and built in such a way as to prevent the dangeroussituations. The following information requires a particular attention: - the machine must not startunexpectedly, - the parameters of the machine must not change in an uncontrolled manner, because such a change can lead to dangeroussituations, - the machine must not be prevented from turning off if the stopping command has already beengiven, - no moving part of the machine or processed piece must fall or be thrownout, - the manual or automatic stop of moving parts of any kind must not beprevented. The machinery must be equipped with a control device that allows the general shutdown in safeconditions The machine stop control must take priority over the start controls. Once the machine or its dangerous operation has been stopped, the power supply of the corresponding actuators must be cut. Standards: ISO/TR 14121-2; EN ISO 12100, EN ISO 13849-1, IEC/EN 62061 and EN 60204-1 Scheduled action: System shutdown by means of emergency button and inspection of the proper operation. Outcome: OK Date: 08/8/2025 Notes: Inspection of the correct online operation. Possible intervention method: Check the correct operation after putting into service the partly completed machine. Closing date: ……………………………………………………… | 4. Selection of the control or operating mode The selected control or operating mode must have priority over all other control or operating modes, except for the emergency stop. Each selector position, which must be clearly identifiable, must correspond to a single control or operating mode. Placed on the switchboard, the selector allows the operation of the partly completed machine in automatic running condition if the system is online or in manual mode (maintenance), in case of a system outside the production line. Standards: ISO/TR 14121-2; EN ISO 12100, EN ISO 13849-1, IEC/EN 62061 and EN 60204-1 Scheduled action: Inspection of the correct operation. Outcome: OK Date: 08/8/2025 Notes: Possible intervention method: Check the correct operation after putting into service the partly completed machine. Closing date: ……………………………………………………………… |
| --- | --- |

| 5. Electrical specifications The machine is powered with electricity, therefore it must be designed, built and equipped in such a way as to prevent or to allow the prevention of all hazards caused by electricity. Standards: ISO/TR 14121-2; EN ISO 12100 and EN 60204-1 Scheduled action: Inspection of the correct operation (mechanical and electrical inspections) Outcome: OK In compliance with standards and regulations. Power voltage according to specifications. Wiring and devices selection according to specifications. Operating conditions followed. Date 08/8/2025 Notes: Possible intervention method: Closing date: ……………………………………………………………… | 6. Noise The machinery must be designed and built in such a way as to reduce to the minimum level the risks caused by the emission of airborne noise Standards:ISO/TR14121-2;ENISO12100ISO3743-1;ISO3743-2;ISO3744; ISO 3745; ISO 3746; ISO 3747; EN ISO 11200; EN ISO 11201; EN ISO 11202;EN ISO 11203; EN ISO 11204; EN ISO 11205; EN ISO 11688-1 and EN ISO 4871 Scheduled action: Inspection of the sound pressure lower than 80 dB (A) by using a sound levelmeter. Outcome: OK Date: 08/8/2025 Notes: Possible intervention method: Closing date: ………………………………………………………… |
| --- | --- |

| 7. Information and information devices The information necessary for operating the machine must be provided in a clear and easily understandable way. The amount of information must not exceed the operator’s ability of processing and understanding. Standards: ISO/TR 14121-2; EN ISO 12100; EN 894-1; EN 894-2; EN 61310-1; EN 61310-2; EN 61310-3; EN 842 and EN 981 Scheduled action: Supply of manuals and all the required documentation in ITALIANO TURCO (as specified) to accompany the machine. Outcome: OK Date: 08/8/2025 Notes: Possible intervention method: Closing date: …………………………………………………………… | 8. Machine labeling Each machine must be marked, in a visible, legible and indelible manner, at least with the following information: - company name, full address of the manufacturer and, where relevant, of his authorizedrepresentative, - machinedesignation - «CE» marking (see. AnnexIII), - series or typedesignation, - if necessary, serialnumber, - year of manufacture, i.e. the year in which the manufacturing process was completed Standards: ISO/TR 14121-2; EN ISO 12100 and EN82079-1 Scheduled action: Application on the switchboard of the CE plate showing the data according to current legislation. Outcome: OK Date: 08/8/2025 Notes: Possible intervention method: Closing date: …………………………………………………………… |
| --- | --- |

  - Signs

The machine is equipped with all the necessary signs to signal the hazards, where present, and any indications for maintenance andcontrol.

## Standards: ISO/TR 14121-2; EN ISO 12100; EN ISO 13849-1; IEC/EN 62061 and

EN 60204-1

Scheduled action: Signs application according to current legislation. Visual inspection Outcome: OK

Date 08/8/2025

Notes:

Possible intervention method:

Closing date: …………………………………………………………

  - Electrical componentsinspection

| Component | Schedule method | Outcome: | Running hours | Possible intervention method |
| --- | --- | --- | --- | --- |
| Tanks level switches | Inspection of the correct electrical and mechanical operation | OK |  |  |
| Teach-in System | Inspection of the correct electrical operation | / |  |  |
| Motor Efficiency In-Out Machine System | Inspection of the correct electrical operation | / |  |  |
| Mixer | Inspection of the correct electrical and mechanical operation | / |  |  |
| Efficiency Single oil solenoid valves | Inspection of the correct electrical operation | OK |  |  |
| Efficiency Heating manifold system | Inspection of the correct electrical operation | / |  |  |
| Efficiency motor Brush | Inspection of the correct electrical operation | / |  |  |
| Efficiency Cyclon System | Inspection of the correct electrical operation | / |  |  |
| Efficiency Motor Suction Brush Unit | Inspection of the correct electrical operation | / |  |  |
| Efficiency motor suction system lubrication machine | Inspection of the correct electrical operation | / |  |  |
| Motor Conveyor system inspection | Inspection of the correct electrical operation | / |  |  |
| Flow Control Nozzle | Inspection of the correct electrical operation | OK |  |  |
| Lifting motor upper spray head | Inspection of the correct electrical operation | / |  |  |
| Safety switch | Inspection of the correct electrical operation | OK |  |  |
| Efficiency main air pressure regulator | Inspection of the correct electrical operation | OK |  |  |

  - Pneumatic and oil-hydraulic componentsinspection

| Component | Schedule method | Outcome: | Running hours | Possible intervention method |
| --- | --- | --- | --- | --- |
| Oil pump | Inspection of the correct Pneumatic and hydraulic operation | OK |  |  |
| Refill oil pump | Inspection of the correct Pneumatic and hydraulic operation | OK |  |  |
| Ingromat pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| Recovery tank pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| 2nd Oil pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| 2nd Refill oil pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| 3th Oil pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| 3th Refill oil pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| 4th Oil pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| 4th Refill oil pump | Inspection of the correct Pneumatic and hydraulic operation | / |  |  |
| Solenoid valve | Inspection of the correct Pneumatic and electrical operation | OK |  |  |
| Wiping system | Inspection of the correct Pneumatic and electrical operation | / |  |  |
| Efficiency Single Spray nozzles | Inspection of the correct Pneumatic and hydraulic operation | OK |  |  |
| Efficiency Single Air valve | Inspection of the correct Pneumatic and electrical operation | OK |  |  |

| Efficiency Upper Ingromat nozzle | Inspection of the correct Pneumatic and hydraulic operation | / |
| --- | --- | --- |
| Efficiency Lower Ingromat nozzle | Inspection of the correct Pneumatic and hydraulic operation | / |

  - Mechanical componentsinspection

| Component | Schedule method | Outcome: | Running hours | Possible intervention method |
| --- | --- | --- | --- | --- |
| Infeed Pinch-roll Height Adjustment | Inspection of the correct mechanical operation | / |  |  |
| Outfeed Pinch-Roll Height Adjustment | Inspection of the correct mechanical operation | / |  |  |
| Efficiency In-Out Machine System | Inspection of the correct electrical and mechanical operation | / |  |  |
| Efficiency Upper Brush | Inspection of the correct electrical and mechanical operation | / |  |  |
| Efficiency Lower Brush | Inspection of the correct electrical and mechanical operation | / |  |  |
| Efficiency Suction Brush Unit | Inspection of the correct electrical and mechanical operation | / |  |  |
| Efficiency Cyclone System | Inspection of the correct electrical and mechanical operation | / |  |  |
| Efficiency suction system lubrication machine | Inspection of the correct electrical and mechanical operation | / |  |  |
| Conveyor system inspection | Inspection of the correct electrical and mechanical operation | / |  |  |
| Control Up/Down crush-less wheels lubrication system | Inspection of the correct electrical and mechanical operation | / |  |  |
| Extraction upper and lower spray box | Inspection of the correct mechanical operation | / |  |  |
| Lifting system upper spray head | Inspection of the correct electrical and mechanical operation | / |  |  |

Date: 08/8/2025

Signature:Tavazzi Davide